# South African Solar PV Compliance Inspection Checklist

(SANS 10142-1:2020 + SANS 10142-1-2 + Electrical Installation Regulations)

## 1. Documentation & Compliance

☐ System design available

☐ Single Line Diagram (SLD) available

☐ PV string calculations completed

☐ Inverter specifications available

☐ Battery specifications available

☐ Manufacturer installation manuals on site

☐ Embedded generation approval from municipality (if grid-tied)

☐ Electrical CoC issued/updated

☐ PV Supplementary CoC issued

☐ Installer registration details available

☐ Equipment carries applicable certification (IEC/NRS where applicable)

## 2. Labelling Requirements

### Main Distribution Board

☐ "WARNING – DUAL SUPPLY" label installed

☐ Main switch clearly identified

☐ Alternative supply warning installed

☐ PV supply breaker labelled

### Inverter

☐ Inverter identification label installed

☐ Shutdown procedure displayed

### Batteries

☐ Battery hazard warning signs

☐ Voltage warning labels

☐ Emergency contact information displayed

### External

☐ PV array warning labels

☐ DC cable route identified

☐ Isolation points labelled

## 3. PV Array Inspection

☐ Modules securely mounted

☐ Roof structure suitable

☐ No damaged panels

☐ No cracked glass

☐ Frames bonded and earthed

☐ Correct orientation according to design

☐ Cable management neat

☐ No cable contact with roof surface

☐ UV-resistant cable ties used

## 4. Roof Mounting Inspection

☐ Mounting system structurally sound

☐ Roof penetrations waterproofed

☐ Stainless steel or corrosion-resistant fasteners used

☐ No signs of corrosion

☐ Wind load requirements considered

☐ Rails properly bonded

## 5. DC Electrical Inspection

☐ DC isolator installed

☐ DC isolator accessible

☐ Correct DC voltage rating

☐ Correct DC current rating

☐ MC4 connectors compatible and matching brand

☐ No mixed connector brands

☐ Polarity verified

☐ String voltages recorded

☐ Cables mechanically protected

☐ DC SPD installed (where required)

☐ String fusing installed (where required)

## 6. Inverter Inspection

☐ Correct inverter sizing

☐ Mounted according to manufacturer requirements

☐ Adequate ventilation

☐ Safe working clearances maintained

☐ Earth connection verified

☐ Firmware updated

☐ No active fault alarms

☐ Anti-islanding protection enabled (grid-tied)

## 7. Battery System Inspection

☐ Battery securely mounted

☐ BMS operational

☐ Correct battery protection installed

☐ DC breaker/fuse installed

☐ Isolation switch installed

☐ Ventilation adequate

☐ No combustible materials nearby

☐ Emergency shutdown available

☐ Battery cable sizing verified

☐ Battery enclosure compliant

## 8. AC Installation Inspection

☐ Dedicated inverter breaker installed

☐ Breaker correctly sized

☐ AC isolator installed where required

☐ Earth leakage protection verified

☐ Neutral-earth arrangement compliant

☐ Cable sizing compliant

☐ Segregation of AC and DC wiring maintained

☐ DB schedule updated

## 9. Earthing & Bonding

☐ PV frame earth continuity verified

☐ Inverter earth continuity verified

☐ Battery earth continuity verified

☐ Earth conductor correctly sized

☐ Earth resistance test completed

☐ Bonding connections secure

## 10. Testing & Commissioning

### Record Values

☐ String 1 Voc ______ V

☐ String 2 Voc ______ V

☐ String 1 Isc ______ A

☐ String 2 Isc ______ A

☐ Insulation resistance ______ MΩ

☐ Earth continuity ______ Ω

☐ Earth resistance ______ Ω

### Functional Tests

☐ Correct polarity confirmed

☐ Inverter startup successful

☐ Grid synchronisation successful

☐ Battery charging operational

☐ Battery discharge operational

☐ Monitoring platform operational

☐ Emergency shutdown tested

☐ System producing expected power

## 11. Insurance Inspection Requirements

Most South African insurers now check:

☐ CoC available

☐ PV Supplementary CoC available

☐ Qualified installer used

☐ Batteries installed to manufacturer specification

☐ Correct protection devices fitted

☐ DC and AC isolators installed

☐ Surge protection installed

☐ Fire risk assessment satisfactory

☐ Installation photographs archived

☐ Asset values declared to insurer

## Final Sign-Off

Site Address: _______________________

Installer: ___________________________

Date: ______________________________

Inspection Result:

☐ PASS

☐ PASS WITH DEFECTS

☐ FAIL

Inspector Name: ____________________

Registration Number: ________________

Signature: _________________________

This is the format many South African solar inspectors and electricians use when preparing for a CoC audit, municipal approval inspection, or insurance assessment.